A Simple Mass Budget of Methylmercury in San Francisco Bay, CA.

Donald Yee, Lester McKee, John Oram

Abstract

The San Francisco Bay is a water body listed as impaired due to mercury contamination in sport fish for human consumption, as well as possible effects on resident wildlife. A legacy of mercury mining in local watersheds and mercury used in gold mining in the Sierra Nevada have contributed to contamination seen in the Bay, with additional more recent and ongoing inputs from urban and industrial stormwater and wastewater effluent. Even without continued mercury inputs, it would likely be decades or centuries before ambient mercury concentrations return to pre-industrial levels. However, not all forms of mercury are equally harmful to organisms.  Methylmercury is the species of mercury most directly responsible for contamination in biota, so better understanding of its sources, loads, and processes is sought to identify the best means to reduce impacts. A simple one box model of San Francisco Bay was applied to evaluate uncertainties in estimates for methylmercury loading pathways and environmental processes, to identify major data gaps, and test various management scenarios for reducing methylmercury contamination in Bay waters. External loading pathways considered in the mass budget include methylmercury loads entering via atmospheric deposition to the Bay surface, and discharges from the Sacramento/San Joaquin Delta, local watersheds, industrial and municipal wastewater, and fringing wetlands. Internal processes considered include exchange between bed and suspended sediments and the water column, degradation, in-situ production, and losses via hydrologic transport to the Pacific Ocean. The largest uncertainties and potential approaches for reducing those uncertainties are discussed.

1. Introduction

Mercury is a pollutant of high concern in San Francisco Bay, due to its listing for impairment of beneficial uses such as human fish consumption and potential impacts on survival of resident wildlife. As a result, a Total Maximum Daily Load (TMDL) process has been undertaken to address the impairment. Mercury has been introduced to the Bay environment through historic gold and mercury mines in California, as well as through ongoing inputs from global and local combustion and other discharges. The Regional Monitoring Program for Water Quality (RMP) in the San Francisco Estuary has monitored mercury in the Bay since its inception in 1993.

The presence of mercury in the environment is necessary but not alone sufficient to cause negative impacts on biota, as mineral and elemental forms of mercury are less bioaccumulative than methylmercury. The RMP has monitored methylmercury in the Bay since 1999, finding that it typically constitutes a very small proportion (average <1%) of total mercury in the San Francisco Bay ecosystem. Therefore, any strategies for managing negative impacts of mercury benefit from improving the understanding of methylmercury processes.

Concentrations of total mercury in the Bay are expected to slowly decline as new uses of mercury decrease over time. However, even without any new inputs, it will likely be decades if not centuries before current ambient concentrations (2002-2006 Bay-wide average ~0.23 µg/kg) return to pre-anthropogenic levels of around 0.08 µg/kg (Conaway, Watson et al. 2004). The RMP has developed a “Mercury Strategy” to determine if it may be possible to identify the specific fractions or sources of mercury entering or already in the Bay that contribute disproportionately to accumulation in species of concern (as indicated by the TMDL targets – sport fish, small fish, avian eggs). If this premise is correct, then it may also be possible to reduce mercury accumulation in species of concern in a significantly shorter period than would be possible via total mercury reductions.

The RMP has already conducted a substantial amount of monitoring to better understand distributions, loads, and effects of mercury and methylmercury, with more information collection planned in the next several years. Objectives of this mass budget exercise are to (1) collate information on methylmercury distributions in the urbanized San Francisco estuary; (2) quantify sources including methylmercury concentrations and loads in atmospheric deposition, urban stormwater, Delta outflow, wetlands, municipal wastewater, and other discharges; (3) develop an annually averaged one-box mass balance for San Francisco Bay using empirical data on environmental processes collected locally where possible, or otherwise use information from the literature for other regions. This is a first step towards developing a better understanding of the factors most likely controlling methylmercury concentrations on the Bay-wide scale. Much of the available information is from intensive studies with limited temporal and spatial distribution, so potential pitfalls of a simplified one-box mass budget are: 1) extrapolation of limited data to wider temporal and spatial scales than appropriate, and 2) under-interpretation of finer details of spatial and temporal data critical to local processes. These limitations will be discussed and may be addressed in future work.

2. Methods

2.1 Location and Physiography

San Francisco Bay, California, USA (Figure 1), receives water, sediments and pollutants from local watersheds, as well as from the Sacramento/San Joaquin watershed (commonly called the Central Valley), which covers an area of 154,000 km2 (~37% of California). Local watersheds draining directly to San Francisco Bay account for an additional 6,650 km2 , with a mix of urban (35% of the land area), agricultural, and open space (e.g. park and other undeveloped) land uses. The volume of San Francisco Bay is approximately 5.5 km3 with an approximate surface area of 1,100 km2 at mean sea level. In addition, a discontinuous fringing marsh of 950 km2 (greatly reduced from the historical extent) occupies the area between uplands and the open Bay. Tides in the Bay are semi-diurnal with a range (MLLW to MHHW) of 1.78 m at the Golden Gate Bridge, varying in magnitude in various parts of the Bay. Average water discharge for the period 1971-2000 from the Sacramento/San Joaquin watershed past Mallard Island was 24.9 km3 annually (McKee et al., 2006). Another 1.05 km3 of freshwater input is provided by the local watershed drainages in the nine-county Bay Area. Suspended sediment loads entering the Bay average 1 billion kg/year (McKee, Ganju et al. 2006). There is no recent estimate of suspended sediment loads entering the Bay from local tributaries; the latest best estimate was 0.75 billion kg/year (Krone 1979) although the RMP is working on refining this estimate (Lewicki and McKee in preparation remove as ref if not ready at press time). The nine-county Bay Area population reached 6.78 million in the 2000 U.S. Census and is growing at a rate of about 5% a year (ABAG 2007). Another 6 million people reside in the Central Valley, upstream of Mallard Island. Some of the larger industrial facilities in the Bay area include oil refineries, a cement plant, an automobile plant, steel manufacturing and fabrication, and the computer and electronics manufacturers.

2.2 Environmental Monitoring

The Regional Monitoring Program for Water Quality (RMP) in the San Francisco Estuary has conducted annual monitoring of open water areas in San Francisco Bay since 1993. Additionally, RMP sponsored pilot and special studies, such as the Estuary Interface Pilot study, the atmospheric deposition pilot study, and the Mallard Island, Guadalupe River, and Hayward Storm Drain (Zone 4 Line A) loading studies have examined pollutant concentrations and loads entering the Bay at specific locations. At the start of the RMP (1993), only total mercury was measured in collected samples Since 1999, measurements of methylmercury have been added to water, sediment, and in some cases, tissue analyses. Other projects and programs have monitored other locations and elements of the ecosystem (detailed as needed in sections below), information which can be used in building our understanding of environmental processes in San Francisco Bay.

2.2.1 Bay Ambient Water and Sediment

Although the RMP has been monitoring Hg in the Bay since its inception in 1993, methylmercury measurements in water and surface (0-5cm) sediment have only been included since 1999. Samples were collected during the dry season (summer) at fixed locations from 1999 to 2001 and primarily at probabilistic sites with some fixed locations since 2002. Historical fixed locations were located along a transect primarily following the deep channel spine of the Bay. Probabilistic sampling locations were selected based on the Generalized Random Tessellation Stratified design (Stevens and Olsen 2000) used by the U.S. EPA’s Environmental Monitoring and Assessment Program. The sampling design allows for the determination of a spatially unbiased estimate of mean contaminant concentrations in water and sediment. Water was collected via peristaltic pump from ~1m depth as either total (unfiltered) or dissolved (filtered, 0.45 µm nominal pore size) samples and frozen in the field. Sediment samples were collected using a modified Van Veen grab sampler, with surface (top 5 cm) sediments composited in the field and immediately frozen.

2.2.2 Sacramento – San Joaquin River Delta

A methylmercury Total Maximum Daily Load (TMDL) report has been prepared for the Delta (Wood et al., 2008) by the Central Valley Regional Water Quality Control Board. Part of that exercise required an estimate of net methylmercury exported from the Delta to the San Francisco Bay. In the Central Valley TMDL, a major export site of water from the Delta is through the channel cross section adjacent to Mallard Island roughly the boundary of the Central Valley and San Francisco Bay Water Board jurisdictions. To obtain export load estimates, methylmercury concentrations were measured at X2, the location in the estuary with 2-o/oo bottom salinity (which can range up to ~10 miles upland or seaward of Mallard Island), or at Mallard Island in more recent studies. Central Valley Water Board staff monitored monthly aqueous methylmercury at X2 from March 2000 to September 2001 (Foe, 2003) and from April to September 2003. Concentrations at Mallard Island were measured in a subsequent CALFED study. Net daily Delta outflow water volumes were determined by the DAYFLOW model (I.E.P. 2008).

2.2.3 Local Watersheds

A number of local tributaries with a mix of urban and other land uses have been monitored by the RMP for total mercury and other pollutants, starting with monitoring of the mining contaminated Guadalupe River in Water Year 2003, continued with a mix of funding through to Water Year 2006. In Water Years 2005 and 2006, methylmercury was also analyzed in samples collected from the Guadalupe River main stem. In Water Year 2006, methylmercury was measured in several mining tributaries to the Guadalupe River (Alamitos Creek and McAbee Creek) and in the San Pedro Road urban storm drain in old town San Jose. The Guadalupe River watershed area below reservoirs is 236 km2, with 13% industrial, 13% commercial and 58% residential land use. Sampling began at the Hayward storm drain in Water Year 2007. Zone 4 Line A was selected for monitoring in recognition that the Guadalupe River watershed is not representative of the smaller urban drainages on the Bay margin where there are no mercury mines and which are more heavily industrialized, have greater commercial land use and have almost 100% urban land use designation. Samples typically were collected over the course of storm events and on a few occasions during base flow. Concentrations in Guadalupe River and Zone 4 Line A have been reported previously (McKee et al., 2004, 2005, 2006; McKee and Gilbreath, 2008). Few to no measurements have been taken for other watersheds, so estimates of combined watershed loads for the region are made by extrapolation of methylmercury percentage from the measured watersheds and regional estimates for Hg loading, with acknowledgement of potentially high uncertainty given differences in watershed characteristics and Hg sources.

2.2.4 Municipal Wastewater

The San Francisco Bay Regional Water Quality Control Board (SFBRWQCB) recently requested information on methylmercury concentrations from local municipal wastewater dischargers over the course of the past year (2007-2008). Using clean techniques, dischargers collected monthly effluent grab samples and reported discrete and annual averaged concentrations. These average concentrations were combined with annual discharge rates for each of the plants to estimate annual methylmercury loads. The reporting municipal dischargers account for 95% of the regional effluent discharge.

2.2.5 Wetland concentrations

Concentrations of methylmercury in wetland waters have been reported in a number of studies. Studies in wetlands of the North Bay (e.g., Petaluma Marsh (Yee, Collins et al. 2008); Suisun Marsh (Yee, Collins et al. 2008)), have reported water column concentrations on incoming and outgoing tides for specific events, but comprehensive monitoring across many days over spring/neap cycles in multiple seasons has not been performed due to the prohibitive field and laboratory effort this would entail. Data on maximum water column concentration differences between incoming and outgoing tides measured in the Petaluma study, and estimates of leachable methylmercury produced in the Hamilton Army Air Field wetland (Best, Fredrickson et al. 2005) can be used to provide an estimate of wetland methylmercury discharge, albeit with large uncertainty. However, such first order estimates based on existing information would indicate whether wetland discharges are a potential source of concern given the range of concentrations and loads found thus far in the sparse data available.

2.3 Mass Budget Model

A one-box model of water and sediment processes was employed to integrate existing monitoring efforts and to enhance our understanding of methylmercury fate in San Francisco Bay. The model was initially developed by Davis (Davis 2004) to predict the long-term fate of PCBs in San Francisco Bay and has been used for developing mass budgets for PAHs, organochlorine pesticides, and PBDEs (Greenfield and Davis 2005; Leatherbarrow, David et al. 2006; Oram, McKee et al. 2008). The one-box model of San Francisco Bay treats the Bay as two well-mixed compartments representing the water column and surface sediments. Conceptually, the model ignores differences in the geographic sub-regions of the Bay, a simplification that precludes deeper understanding of temporal and spatial variations, but allows a first-order evaluation of the system. The model includes parameters for describing major physical and chemical processes governing the transport and fate of contaminants in aquatic systems. These processes include:

- external loads,

- settling and resuspension of sediment particles,

- water-solid partitioning (sorption/desorption),

- sediment-water column diffusive exchange,

- volatilization,

- degradation in water and sediment,

- tidal flushing/outflow,

- in-situ production

In-situ production is a major component missing from the earlier one-box models, as it is negligible for the previously modeled organic contaminants. In-situ production is critical for understanding methylmercury given its facile transformation to and from inorganic forms. A previous effort for a mass balance of Hg in the San Francisco Bay treated these transformations as a pseudo-equilibrium characteristic of the system, using a fixed percentage of total Hg (MacLeod, McKone et al. 2005) to model a pseudo-steady state methylmercury concentration. However, in this model we do not attempt to predict the concentration or production rate of methylmercury as a function of total Hg concentration. Instead, methylmercury production is treated as a function only of the volume of sediment considered to be a methylating zone in the ecosystem, as data collected in the RMP to date have not shown any significant trend in total mercury concentrations. A rate calculated based on total mercury would become a de facto constant methylmercury production rate, unless it was linked to a concurrent model of long-term total mercury mass balance. Linking of mercury and methylmercury fate models was beyond the scope of this simple modeling exercise, whose main objective was to determine the prospects for ecosystem recovery by managing methylmercury rather than total mercury, and to identify the key factors and uncertainties for methylmercury fate in San Francisco Bay.

2.3.1 External Loads

In the model, some inputs to the Bay are external and not dependent on the state of the Bay; these are handled as fixed magnitude inputs entering from the air via direct deposition or from the land via rivers, tributaries, channels, and discharge pipes. These external loads are discussed below, and summarized in Table 1. Although flows and loads (e.g. from precipitation runoff in the Delta and local watersheds) are not uniform over the course of the year or evening distributed in space, this model simplifies temporal loads variability by treating inputs as occurring uniformly throughout the year and simplifies spatial heterogeneity by assuming all loads enter the Bay evenly from air and margins (one well-mixed box). These simplifications are extremely important and will be discussed in detail later.

2.3.1.1 Sacramento – San Joaquin River Delta

In the Delta methylmercury TMDL ((Wood, Foe et al. 2008) average annual methylmercury exports were estimated for water years (WY) 2000 to 2003, using the available information for that period (relatively dry years). Concentration data for samples collected at X2 from two sampling periods (March 2000 to September 2001, and April to September 2003) were combined to derive monthly average concentrations, and the DAYFLOW program was used to derive average monthly flows. Methylmercury concentrations at X2 ranged from below detection limits to 0.241 ng/L, averaging 0.075 ng/L over all periods combined. Monthly average concentrations were multiplied by monthly average flows for WY2000-2003 to estimate monthly loads. Monthly loads were summed to calculate an annual average methylmercury load of 1.7 kg/year (4.7 g/day). A simple model regressing Delta outflow to methylmercury concentration for that period resulted in a similar estimate, with export of 2.1 kg/year. Given their similarity, the monthly average derived export was used in the TMDL mass balance.

A subsequent CALFED funded study measured methylmercury concentrations at both Mallard Island and at X2 from October 2004 to November 2005 to examine differences between estimated fluxes for the two locations, but no significant differences were found. Combining previous data used in the TMDL with the new data resulted in an estimated average export flux of 9.8 g/day (Foe, Louie et al. 2008 in review)The previous TMDL export calculation only attempted to estimate advective flux from the Delta, but dispersive flux could also affect net export.Ignoring dispersive transport results generally in an overestimate of advective flux, averaging nearly 15% for periods of Delta outflow over around 500 m3/s (McKee, Ganju et al. 2006)  The error in estimated mercury advective flux would be expected to be of a similar magnitude, but even with correction for dispersive flux, uncertainty in average annual flux is larger yet (~30/%). Potential impacts of all these uncertainties can be assessed together through sensitivity testing of the estimate for Delta flux in combination with other external loads.

2.3.1.2 Local Watersheds

Concurrently collected samples were analyzed for methylmercury and total Hg in the Guadalupe River and Hayward Zone 4 Line A watersheds for multiple events in one or more rainy seasons as part of RMP special studies of local watershed loads. Other locations near Alamitos and McAbee mining impacted Creeks and in San Jose (San Pedro storm drain) have been sampled more sparsely . Guadalupe River total (combined dissolved and particulate phase) methylmercury concentrations ranged 0.05-2.2 ng/L (average 0.7 ng/L), which was an average 0.45% of total Hg concentrations (range 0.05-1.9%). Total methylmercury concentrations in Hayward Zone 4 Line A samples ranged from 0.08-1.3 ng/L (average 0.44 ng/L), averaging 1.6% of total Hg in water. Other urban sites around San Jose had similar concentrations, with San Pedro storm drain concentrations ranging 0.02-3.1 ng/L (average 0.84 ng/L), and a location on Airport Parkway with 0.95 ng/L for a single sampling event. Methylmercury percentages for those two sites were 0.43% and 1.4%, respectively.

Because methylmercury concentrations and percentages (relative to total Hg) are not yet known for other watersheds, estimates for the rest of the region require extrapolation of the limited existing data. Methylmercury percentages in the literature range widely, although percentages reported are generally below 10%. Thus the concentrations and percentages found here in stormwater (methylmercury between 0.02-3 ng/L, averaging around 0.4-1.5% of total Hg, for different land use types) do not seem unreasonable. In RMP ambient monitoring throughout the Bay, methylmercury averages <1% of the total Hg in the water column, and a similar percentage (average 0.7% methylmercury:Hg) was found in monitoring from the Delta to Central Bay in a CALFED study (Choe, Gill et al. 2004). A synoptic study of the upper portions of the Guadalupe River watershed (Tetra Tech 2003) also found similar distributions at the station furthest downstream (Los Gatos Creek near the confluence with the Guadalupe River), where methylmercury was 1.2% of total Hg. Thus average methylmercury loads at around 1% of total Hg loads are likely to be a reasonable first order estimate. The problem is that total mercury loads at a regional scale have been estimated using multiple methods (e.g. the SIMPLE model(KLI 2002), or combining bed sediment concentrations with and regional suspended sediment loads estimates (Johnson and Looker 2003). Regional total mercury loads estimates from these previous studies range between 123 kg/year  and 185 kg/year. The corresponding methylmercury loads based on 1% of total mercury loads would be between 1.2-1.9 kg/year.Given the uncertainties associated with estimating methylmercury as a fixed percentage of total mercury described in the previous paragraph, a second method was applied to provide a comparison. A first order estimate of methylmercury can also be developed using the limited existing data for general land use types and the SIMPLE model (Davis, McKee et al. 2000). Mass loads of methylmercury estimated in this manner were 1.3kg/year (0.4 kg/year for urban watersheds, 0.4 kg/year for non-urban watersheds, and 0.5 kg/year from the Guadalupe River alone). Using the SIMPLE model to estimate total mercury loads for different watershed types in the region and multiplying by average methylmercury percentages for those land use types results in slightly (~3x) higher methylmercury load estimates, with about 1.5 kg/year from urban watersheds, 1.3 kg/year from non-urban areas, and 0.5 kg/year from the Guadalupe River. Summing these gives a total estimated annual average methylmercury load entering the Bay of 3.3 kg/year. Given there is no way of knowing which is the more accurate of these estimates, to the best alternative is to use a load between the two described here and the one described in the previous paragraph. Thus, a load of 2.3 kg/year was chosen as the best approximation of local watershed loads for the mass budget presented here.

Given the limited available data, there is substantial uncertainty in the possible range of methylmercury concentrations and percentages locally, with a similar degree of uncertainty in the range of Hg concentrations. In acknowledgement of this uncertainty, the mass balance model developed here was tested over a one order-of-magnitude range of loads for local watersheds. This approach allowed for the evaluation of the importance of local watershed loads relative to other internal and external methylmercury inputs.

2.3.1.2 Wastewater

Monthly methylmercury data from the 16 largest treatment plants were compiled, and combined with data on mean annual discharge volume (Larry Walker Associates (LWA) 2002) to estimate mean annual methymercury loads. These plants account for around 95% of wastewater discharged to the Bay, yielding a total methylmercury load of 0.8 g/day. Concentrations at treatment plants were highly variable (mean RSD ~65%) he so a large range of possible loads was used for the model sensitivity runs.

2.3.1.2 Wetland Discharge

One approach to calculating net import or export from tidal wetlands is to measure concentrations and flows in the water column during flood and ebb tides. Neither currents nor methylmercury concentrations are uniform over the course of a tidal cycle, and most wetlands have multiple inlets and outlets, so many measurements at many locations would be needed to get an accurate estimate of net transport for even a single wetland. Given the paucity of data on net import or export on the numerous wetlands fringing the Bay, simplifying assumptions and extrapolations from limited data were made to estimate export rates, which can be compared to standing inventories and production rates in wetlands to determine if estimated wetland export rates are reasonable.

The current extent of tidal marsh area in the San Francisco Bay region is about 40,000 acres, greatly reduced from its historical extent of 190,000 due to diking and bayfill (Goals Project 1999). These wetlands vary widely in elevation, vegetation, and hydrological connectivity, but for the purpose of this mass budget exercise, we treat all these tidal wetlands as a single uniform source. Long term tidal data for NOAA benchmark stations around San Francisco Bay (Port Chicago, Mare Island, Richmond, San Francisco, Alameda, Redwood City) show differences between mean high water (MHW) and mean tide level (MTL) averaging 0.7 m. Assuming wetland areas with linear slope, an average of 0.35 m water covers the marsh surface on each high tide (twice daily).

With twice daily tides and an estimated time-averaged depth of 0.35m each tide, an equivalent depth of over 200m of water is transported on and off wetlands annually. Local wetland evapotranspiration rates are estimated to be around 1m/year (Blaney and Muckel 1955), and average annual rainfall around 0.5m/year (WRCC 2008), so water movement via tides swamps other hydrologic transport pathways in most tidal wetland areas.  Although episodic flows from storm events may transport greater volumes for short periods, on an annual basis, transport via daily tidal flows is likely to dominate in wetland areas other than directly along stream banks, which are already counted as tributary flow estimates.

In a wetland near the mouth of the Petaluma River studied in a Calfed-funded project, water column concentrations over 24 hours were monitored to obtain estimates of the net flux of methylmercury (Yee, Collins et al. 2008). Maximum water column dissolved methylmercury concentrations on outgoing tide at that wetland were up to ten times higher than the concentrations seen during the flood tide, with the average ebb concentration (0.136 ng/L) almost double that of flood concentrations (0.083 ng/L). In contrast, particulate concentrations averaged slightly higher during the flood tide (0.098 ng/L) compared to the ebb tide (0.092 ng/L). Extrapolating the difference in dissolved methylmercury between flood and ebb tides, and transport dominated by approximately equal volumes of water transported in and out of wetlands on each tide, methylmercury mass exported from 40,000 acres of tidal marshes around San Francisco Bay would total 6.0 g/day. Similarly, from the differential in particulate methylmercury concentrations between incoming and outgoing tides, about 0.7 g/day of methylmercury would be transported from the Bay to wetlands. Thus net methylmercury transport would be 5.3 g/day exported from wetlands to the Bay.

In a study of the Hamilton Army Air Field wetland, the USACE (Best et al 2005) estimated potential methylmercury export by another methodology. It was assumed that solubility would control the amount of methylmercury transferred from wetland sediments to overlying water. The net methylmercury production rate was estimated to be 3.1 µg/m2/day at HAAF. Based on Hg solubility (Brannon et al, 1980), 0.4% of methylmercury was estimated to be exchangeable with the water column. Combined with tides twice daily, 0.8% of daily net methylmercury production would be removed through tidal transport. Extrapolating the HAAF rates to the total wetland area around the Bay, 4.0 g/day of methylmercury would be loaded to the Bay from wetlands. This export rate is of the same order-of-magnitude as that estimated by the differential in flood and ebb tide concentrations measured at Petaluma.

Studies in Suisun Marsh (Stephenson, Foe et al. 2005) found mixed results in net methylmercury transport, with some studied periods and locations showing net import to the marsh, and others showing net export. Although dissolved and particulate methylmercury were not measured separately in those studies, the authors hypothesized that differences in net transport resulted from differences in hydrology and particulate and dissolved phase methylmercury between events and locations. This is consistent with the findings in the Petaluma wetland, where particulate methylmercury concentrations were higher in flood tides, whereas ebb tides had higher dissolved methylmercury concentrations. An accurate determination of net methylmercury transport between a wetland and the Bay would require monitoring over a long term under a wide range of hydrologic conditions (flood and ebb tides under spring and neap tide periods during wet and dry seasons, and during and after wet season rainfall events of different intensities and durations), a task beyond the scope of most studies, including this exercise. However, the rough estimates provided by studies conducted to date provide a starting point to evaluate the importance of net transport to and from wetlands on the net methylmercury budget of the Bay.

2.3.4 Atmospheric deposition

The majority of atmospheric mercury monitoring under the Mercury Deposition Network measures only total Hg in wet deposition, although sites have occasionally monitored methylmercury concentrations in precipitation. Although an MDN station measuring total Hg was maintained near San Jose for 6 years, methylmercury in precipitation was not measured at that local station. The U.S. Geological Survey, in cooperation with the Indiana Department of Environmental Management monitored both total and methyl Hg in precipitation at several stations in Indiana between 2001-2003 (Risch 2007). For the 3-year period, the median methylmercury concentration in weekly samples was 0.058 ng/L with a maximum of 5.77 ng/L. Methylmercury was also found in precipitation in a similar concentration range (0.01-.0179, average 0.052 ng/L) in the Experimental Lakes area of Northwestern Ontario (St. Louis, Rudd et al. 1995). Rain samples collected during a storm event passing over the North Olympic Peninsula in western Washington State showed average methylmercury concentrations of 0.15 ng/L (Bloom and Watras 1989). Taking the mean methylmercury concentrations in rainfall of these various studies, (0.087 ng/L) and applying San Francisco Bay area mean annual rainfall (typically between 0.4-0.5 m/year), direct wet methylmercury deposition to the Bay is estimated to be 0.1 g/day.

Methylmercury in dry deposition is seldom measured. A recent study characterized dry deposition of methylmercury in the ELA region through collection of throughfall and litterfall, subtracting open field deposition (Graydon, Louis et al. 2008). Although it is possible to measure throughfall and litterfall in surrounding watersheds, dry deposition of methylmercury onto watersheds is already accounted for in the estimation of watershed loadings via tributaries. Measurement of throughfall and litterfall directly to the Bay would not be possible, as the Bay is primarily open water. The combined throughfall and litterfall deposition rates in the ELA study were up to double the rates of wet deposition, so as a first-order approximation, any dry deposition directly to the Bay would likely be a similar order-of-magnitude as wet deposition.

2.3.2 Uptake by biota

Potential methylmercury bio-uptake is modeled as a constant rate based on biota for which relatively good inventories exist to evaluate their impact on the overall mass budget. With sufficient information methylmercury uptake into biota might eventually be modeled using concentration dependent relationships, though currently a lack of good inventories for many types of biota and a lack of sufficiently detailed information on the relationship between various biota and water or sediment concentrations preclude explicit modeling of these processes for the mass balance developed here.

Estimating uptake of methylmercury by primary producers would be a possible starting point towards quantifying bio-uptake of methylmercury, but the rapid turnover rate of phytoplankton (0.2-0.7/day (Cloern, Grenz et al. 1995)) suggests that much of the methylmercury in phytoplankton could be rapidly cycled, with potentially a large fraction returning to the water column and sediment. That fraction would depend on what proportion of phytoplankton was consumed by higher tropic organisms and the methylmercury assimilation efficiency of those biota. Because the primary interest of this modeling exercise is to estimate the fate of methylmercury on annual and interannual time scales, small fish seem a reasonable candidate for estimating annual methylmercury transfer to biota. Small fish are relatively well studied in the Estuary, with the California Department of Fish and Game conducting monthly trawls in the Bay and Delta for the California Department of Water Resources Interagency Ecological Program (I.E.P). The size/age relationship for various species in the Bay have been studied, so the mercury body burden in the young-of-year cohort can be used to represent annual-scale net uptake.

The mean pelagic fish biomass in June-October trawls for areas in northern San Francisco Bay in the intermediate and high salinity areas typically ranged 100-1000 g carbon (g C) per 10,000 m3 trawled (Slater 2007). Using the geometric mean of the year 2000-2006 range (~0.02-0.03 g C/m3), and converting back into wet weight (original data were expressed as wet weight catch per unit effort), we find about 0.17 g/m3 in (pelagic) young-of-year fish biomass. The average mercury wet weight concentration in small fish measured in San Francisco Bay by the RMP was 0.049 µg/g. Applying the young-of-year fish density to the total Bay volume, assuming the uptake rate is uniform over the course of a year, an estimated 0.13 g/day of methylmercury is transferred to small fish biomass each day. The data from mid-water trawls likely under-represents benthic residing fish. However, the mercury mass estimate also likely overestimates pelagic fish in deep waters by applying their density to the entire Bay water volume.

Subsequent otter (bottom) trawl data supplied by CDFG (Slater, personal communication) suggest these errors roughly offset. Combining all the Bay segments for 2000-2006 results in a wet weight average 0.21 g/m3 for demersal fish density. Thus the estimated net biouptake loss of methylmercury to fish is likely to be of the right order of magnitude and is a very small component (~0.5%) of overall methylmercury loads to the Bay.

This fish biomass estimate may also be spatially biased as the mid-water trawls can only sample water 2.5 m or deeper.  The US Fish and Wildlife Service conducts beach seining in waters up to ~1m deep as part of the Delta Juvenile Fish Monitoring Program (U.S.F.W.S. 2008).  Biomass catch per unit effort in that program is about ten-fold higher than in the open-water trawls (around 2 g/m3 ) but waters <2.5 m deep account for around one tenth of the Bay volume.  Including the beach seine data to derive a volume weighted average biomass would result only in a two-fold change in the amount of methylmercury in biomass (to ~1% of MeHg loads)

2.3.3 Process Estimates

A majority of the Bay internal processes in the one-box model are dependant on ambient concentrations in Bay waters and sediments. For processes such as degradation, the magnitude is proportional to the concentration in the modeled compartment. For transport and partitioning, relative concentrations between water and sediment and adjoining compartments such as the Pacific Ocean and the atmosphere are needed. Many of the key model parameters are listed in Table 2 and discussed below.

2.3.3.1 Atmospheric volatilization

Methylmercury can volatilize as the charge neutral species MeHgCl. Air-water partitioning of MeHgCl was measured for 0.7 M NaCl (Iverfeldta and Lindqvista 1982) with a dimensionless Henry’s law constant of ~2×10−5 at 25 °C. The volatilization rate calculated by the model using this constant is likely to be biased high, as the Bay water surface temperature is below 25 °C for most of the year. Furthermore, not all the “dissolved” phase methylmercury in surface waters is present as MeHgCl, as methylmercury may also complex with dissolved organic matter or partition to colloidal material in the operationally defined (<0.45 µm) “dissolved” phase. Binding constants with humic acids in freshwater (Hintelmann, Welbourn et al. 1997) and gel permeation chromatography (O'Driscoll and Evans 2000) suggest that half or more of dissolved methylmercury may be complexed to various forms of DOC. Heavy organically complexed and adsorbed colloidal methylmercury are not volatile, so given dissolved methylmercury concentrations are only partially MeHgCl or similarly light charge neutral species, the rate of volatilization calculated by the model represents an upper bound estimate of the loss rate by this pathway.

2.3.3.2 Degradation in water and sediment

Degradation in the sediment and the water column is an internal loss pathway for methylmercury. Degradation can occur through a number of biotic and abiotic pathways. In San Pablo Bay sediment, oxidative demethylation was posited to be a primary mode of degradation based on the by-products of 14C-methylmercury demethylation experiments (Marvin-DiPasquale, Agee et al. 2003). Sediment first-order degradation rate constants ranged from 0.019-0.25 /day (i.e. 1.9-25% degraded per day) in that study. We applied the geometric mean (0.083 /day) as the rate constant for sediments throughout the Bay. Most degradation rates in that study were determined for surface (0-4 cm depth) samples, but in the one site where degradation rates were measured at multiple depths (8 cm and deeper), degradation rates were up to about 10-fold lower below 8 cm. Therefore for purposes of the mass balance model, we assumed that sediment methylmercury degradation primarily occurred in the top 7 cm of sediment, with negligible degradation in the deeper more anoxic layers.

Methylmercury degradation in the water column also may occur through biotic pathways similar to those in sediments, but abiotic pathways such as photodemethylation are the focus of most degradation studies in surface waters. Degradation rate constants of 0.11-0.22 /day were measured in Delta surface waters (Byington, Coale et al. 2005). Similar rates were seen in photo-irradiated water samples from Petaluma wetlands, with half-lives of 5-20 days for filtered waters, and generally longer half-lives (11-20 days) for unfiltered waters. Petaluma samples nearest San Pablo Bay had the shortest half-lives in filtered samples of 5-6 days, but wetland waters are much shallower than those in most areas of the Bay (typically around 1 m maximum at high slack tide compared to the Bay average of around 5 m).

Waters at shallow depths typically experience higher levels of irradiation and thus show higher rates of photodemethylation (Krabbenhoft, Olson et al. 2002). Shallow Bay surface waters are likely to have a demethylation half-life similar to that seen in the wetland nearest San Pablo Bay of 7 days (first order degradation rate of approximately 0.1 /day). Unlike in the wetlands however, light penetration in much of the Bay is not likely to extend the entire depth of the water column to the sediment surface. Light penetration in northern San Francisco Bay as measured by Secchi disk in three segments (Suisun, San Pablo, and Central Bay) over four seasons, ranging 0.3-1.6 m (Wilkerson, Dugdale et al. 2006). In all but 2 measurements (Central and San Pablo Bay in the fall), average Secchi depths were 1.1 m or shallower. Thus, if we assume that demethylation occurs only over the top ~1 m of surface waters, the demethylation rate applied to the entire water column may be modeled as being five-fold lower (0.02 /day).

2.3.3.3 Tidal flushing/Outflow

Bay-specific model parameters were identical to those used in predicting the long-term fate of PCBs in the Bay (Davis 2004), with the addition of a tidal flushing ratio (α = 3.75), which is the ratio of tidal exchange flow to net freshwater flow in the system. Tidal flushing was not included in the original application of the one-box model to PCBs. Equations governing tidal flushing were added to the one-box model in response to review comments (Connolly, Ziegler et al. 2005) and used in more recent applications of the one-box model such as the PBDE mass balance (Oram, McKee et al. 2008).

Methylmercury concentrations in the water column measured by the RMP at the Golden Gate station outside of San Francisco Bay typically have not been detected (<20 pg/L). For the base case assumption in running the one-box model, ocean waters were assumed to have no methylmercury (concentration of 0 pg/L), which would maximize the estimated net export rate, as methylmercury would only be transported with the ebbing tide, transporting methylmercury from the Bay to the ocean. The model was tested using different oceanic methylmercury concentrations to assess the influence of this assumption.

2.3.3.4 Sediment-water Partitioning

Partitioning of methylmercury between the water and sediment phases is an important characteristic of the system. The sediment-water partition coefficient (Kd) determines the degree to which methylmercury in sediments adsorb to or desorb from particle surfaces. Methylmercury loads introduced via the water column may adsorb to suspended particles and settle to the sediment bed. Conversely, methylmercury produced in sediment may dissolve from bed sediments into porewater or desorb from resuspended particles into the water column. The partition coefficient is treated as the equilibrium ratio (L/kg) between concentrations in solid (µg/kg) and liquid (µg/L) phases. The use of partition coefficients relies on the assumptions that the kinetics of partitioning are fast relative to other processes; for example, the Kd would become elevated as the apparent water column concentration was decreased by biological degradation or hydrologic exchange that removed methylmercury much faster than it could be replenished by desorption.

Kinetics of methylmercury adsorption and desorption are relatively fast, reaching equilibrium on the order of hours (Hintelmann and Harris 2004), compared to the daily time step, hydrologic turnover times on the order of a week or greater, and the annual or longer time scale scenarios being modeled. Thus although equilibrium assumptions may not be strictly correct, they suffice to reasonably characterize the partitioning of the system for the one-box model. Using the Bay-wide mean particulate methylmercury concentration in water (49.6 pg/L), the Bay-wide mean dissolved methylmercury concentration in water (44.5 pg/L), and the Bay-wide mean concentration of suspended particles in water (0.085 g/L), the methylmercury partitioning coefficient is estimated to be 13,100 L/kg (log Kd=4.11), consistent with other measurements in the San Francisco Estuary (Choe and Gill 2003) and similar to that reported for the Guadalupe River (McKee, Oram et al. 2006). If instead the average bed sediment methylmercury concentration (0.558 µg/kg) is used, the resultant Kd between sediment and the water column is 12,500 (log Kd = 4.09), virtually the same. A value of 13,100 L/kg was used as the water column Kd for the base case in the model, with higher and lower partition coefficients used for sensitivity analysis.

The partitioning of methylmercury in sediment porewater may potentially differ from that in overlying water column due to differences in various factors that affect its solubility such as organic carbon concentrations and sulfide speciation. Work in northern San Francisco Bay for a CALFED project examined sediment and porewater concentrations of methylmercury and derived Kd for those sites (log Kd = 4.7 ± 0.4 (average ± stdev)). This translates to a mean porewater Kd of 45,700, about the same order-of-magnitude as the partition coefficient for the water column as estimated above.  A porewater Kd of 45,700 (log Kd = 4.7) was used for the base case model scenario.  Effects of higher and lower porewater Kds were tested during sensitivity analysis.

2.3.3.4 Sediment-Water Column Particle Exchange

The exchange of particles between the water column and the bed sediment of the Bay is not spatially uniform and highly dependant on many environmental factors, such as tidal currents, water depth, wind waves, and particle export from local watersheds and the Delta. The one box mass balance model does not capture such spatial heterogeneity, and given its simplifying assumptions it is best suited to simulating a simple system achieving a pseudo-steady state condition, rather than dynamically modeling episodic or transient events. Although the simplifying assumptions of uniform mixing and equilibrium are not accurate synoptic representations of the system at any particular time, in the longer term, the system will tend to range around a mean, which might be reasonably modeled as a steady state condition so long as the kinetics of the modeled processes are much shorter than the modeled period. The similarities in suspended sediment and bed surface sediment methylmercury concentrations suggest that a model of continuous exchange between these two comparments is reasonable.

One major simplification was the treatment of suspended sediment concentration as approximately constant.  Although there has been a long-term trend toward reduced sediment loads coming from the Delta in recent decades (Krone 1996; McKee, Ganju et al. 2002), given the high turnover rate of other modeled processes compared to the long-term change in SSC, modeling of water-sediment particle exchange was simplified and treated as a steady state (i.e. all inputs roughly equal all losses).  Another major simplification is the treatment of the mixed sediment layer as a uniformly mixed compartment. In the case of conservative pollutants, a uniform mixing assumption tends to accelerate the response of the system to changes in loads, as increases or reductions in loads will be instantly modeled as occurring equally throughout the Bay. However, methylmercury is not a conservative pollutant; given turnover times on the order of days for some processes such as degradation, the impacts of an instantaneous mixing assumption are lessened. Although the system response time to changes will be shortened by the uniform mixing assumption, the resulting steady state mass achieved should be essentially the same in the long term.

2.3.3.5 Sediment-Water Column Porewater Exchange

In addition to exchange of particulate matter between the sediment and water column, exchange of porewater with overlying water is another pathway for methylmercury transport. Porewater exchange can occur through abiotic processes such as diffusion or resuspension by wind-wave processes, or through biologically mediated processes, such as bioirrigatation and bioturbation by benthic organisms. Flux rates may be empirically determined via flux box measurements or through mesocosm experiments, although each approach presents limitations which may lead to differences in net flux from actual in-situ condition. For example, mesocosms, even if transferred intact, are disconnected from the larger ecosystem and thus may not entirely reflect native conditions. Unless processes such as wind-wave and tidal current shear stresses, tidal pumping, and groundwater flow are adequately simulated, those components of transport will not be included. Similarly, flux box experiments, although maintaining some connection with the surrounding system, tend to isolate the studied patch from waves and currents, and may trap or (more likely) exclude mobile macrofauna, leading to unrepresentative rate measurements.

Nonetheless, in-situ flux box experiments likely represent the best available measurement of actual fluxes in the native ecosystem, and provide a reasonable likely lower bound estimate of net flux due to possible exclusion of some abiotic forces such as resuspension by waves and stronger currents. Flux measurements were made in benthic chamber deployments in Suisun Bay and the Delta (Choe, Gill et al. 2004). The benthic chambers were 
“gently stirred”, which reduced the boundary layer at the sediment-water interface, but likely were not sufficient to resuspend bed sediments. The median flux rate measured was 13 ng/m2/day (10-90th percentile range 2-55 ng/m2/day). Applying that flux rate to the Bay surface results in a net flux of 14 g/day. However, the Bay average concentration differences in porewater and overlying water may not be the same as in the work by Choe and collaborators, so concentrations and measured fluxes were combined to estimate a transfer velocity (Vd) applied in the form:

Flux (ng/m2/day) = Vd (m/day) * [Csed –Cwater] (ng/m3)

where Csed and Cwater are dissolved methylmercury concentrations in the sediment and overlying water, respectively. The resultant Vd estimate was 0.001 m/day, which is used to parameterize the one-box model.

Although they did not measure methylmercury flux, the U.S. Geological Survey measured total mercury flux in a number of South Bay sites (Topping, Kuwabara et al. 2004). Average dissolved Hg fluxes ranged from 100-400 ng/m2/day. Assuming porewater ratios of methylmercury:Hg similar to those for the sediment (~1% or less), corresponding methylmercury fluxes would be around 1-4 ng/m2/day, about the same range as found in the North Bay/Delta study of Choe and collaborators. This provides another estimate for methylmercury flux and additional verification that the parameters for estimating benthic flux are reasonable when considered in the larger context of other available data for the Bay.

2.3.3.6 Sediment burial or erosion

Net burial or erosion of bed sediments could also result in methylmercury loads to or losses from the mixed sediment layer. In part due to the relatively rapid sediment degradation rates noted previously, methylmercury concentrations found at depth measured in samples from this region were lower than those in surface samples (Marvin-DiPasquale, Agee et al. 2003). Thus unlike the persistent organic pollutants such as PCBs and the inorganic metal pollutants such as mercury or copper, there are not likely to be substantial deeper legacy deposits of methylmercury with higher concentrations that can become exposed through erosion and re-introduced into the ecosystem.

Net sedimentation leading to burial of below the active sediment layer can remove methylmercury from exposure to biota. The average net sedimentation rate of 0.83 cm/year for a core taken from Richardson Bay in 1992 (Fuller, van Geen et al. 1999) would indicate that less than 10% of the methylmercury inventory in the 10cm mixed sediment layer of the one-box model would be lost by burial each year. Another core in that study from San Pablo Bay suggested a much higher burial rate, averaging 4 cm/year. However, comparisons of bathymetric change suggest that such high burial rates are atypical for much of the Bay (Jaffe, Smith et al. 1998; Cappiella, Malzone et al. 1999; Jaffe and Foxgrover 2006), with most areas showing no net bathymetric change or slight erosion (typically <0.5 m in 30-40 years) in recent history. However, even assuming such high burial rates were widespread, losses of 40% of the methylmercury in surface sediments over a year are likely dwarfed by potential losses of a similar magnitude in just a week via degradation in sediments. The model response to changes in the burial rate and other assumptions are discussed below.

3. Results and Discussion

3.1. Model Hindcast

Like the PCB and PBDE mass budget models (Davis 2004; Oram, McKee et al. 2008), the methylmercury mass budget was run in both hindcast and forecast modes. A hindcast is most useful when there is sufficient data to parameterize the model historically, and the modeled pollutant is sufficiently persistent that prior condition is a significant factor in determining the current state of the system. Unlike for PCBs or PBDEs, little or no information exists regarding the history of likely changes in global or local emissions and loading rates for methylmercury, nor of large changes in partitioning, production, degradation or other processes. Thus, the hindcast scenario was made identical to the forecast aside from the initial condition. The sensitivity of the model to historical conditions was tested by adjusting the initial ambient concentrations. Current loading and process rates in the model have no linkage to the historical rates other than through their dependence on ambient concentrations. In the hindcast, to bound the possibilities given no knowledge of the prior condition, we either assumed that ambient methylmercury concentrations in water and sediment were zero, or that they were an order of magnitude higher than the current condition.

The zero initial concentration condition represented the hindcast best case scenario, essentially assuming that prior to any anthropogenic Hg releases, ambient methylmercury inventories were negligible. The model was run for two years with continuous loading rates and internal processes using contemporary rates and coefficients until a steady-state methylmercury mass was achieved. The modeled system reached steady-state quickly, stabilizing in a fraction of a year (Figure 2). The opposite case, with the initial condition set at 10 times the current ambient inventory, showed a similar response, with the system again quickly reaching steady state in well less than a year. The final steady state masses of methylmercury in water and sediment compartments were essentially identical between the scenarios. This indicated that prior condition had negligible influence on determining the methylmercury steady-state inventory of the modeled Bay system.

3.2 Base Case Forecast

As stated previously, no data exist indicating significant changes in various methylmercury or in rates and coefficients for various processes. Thus, the same rates and coefficients for these parameters were used in the model forecast and hindcast. In forecast mode, the model was initialized with the best estimate of the current methylmercury mass in the Bay and loading rates of between 2.7 and 24 kg/yr (from 1/3 to 3x the base case) were examined. Given the same final steady state under both the low and high concentration initial condition assumptions in the hindcast runs, not surprisingly, the base case scenario arrived at a final state near the same final inventory (Figure 3). Adjustment of loading rates over a 9x range had little effect, with the sediment inventory virtually unchanged, and projected water inventories nearly within the 95% confidence interval of the current mean.

The magnitudes of various methylmercury environmental process rates (kg/day) are listed along with the base case steady state inventory (kg) in Table 3. For the sediment, methylation and demethylation respectively produce and remove nearly 6% of the Bay sediment methylmercury inventory each day. In contrast, the net methylmercury moved by the next largest pathway, sediment accretion leading to net burial, at 0.007 kg/day, was around 250 times smaller. Sediment exchange with the water column was a similar magnitude, 0.007 kg/day. The net exchange was mostly the result of water column suspended particle settling (0.038 kg/day methylmercury) combined with bed sediment resuspension (0.045 kg/day methylmercury). The mass of sediment settling out and resuspended to the water column each day were roughly equal in the base case, so the differences in downward versus upward flux were primarily due to differences in methylmercury concentrations of suspended versus bed sediments.

For methylmercury in the water column, the largest inputs are external loads (including atmospheric deposition, Delta, local tributary, wastewater, and wetlands discharge), and exchange with the bed sediment, which are of roughly the same magnitude in the base case. At steady-state, these inputs are nearly offset by degradation in the water column and outflow from the Bay, which are also of similar magnitudes. The other loss pathways included, biouptake into fish and volatilization of methylmercury, are so small as to be negligible in the mass budget.

Although the Bay is not truly a steady-state system, gross deviances in the model steady-state from the initial condition (derived using mean concentrations) would suggest major errors or uncertainties in some of the model parameters and/or assumptions. For the sediment, the model methylmercury inventory at the final steady state (30.8 kg) is similar to the initial ambient condition inventory set using averaged monitoring data (30.7 kg). Given that the initial inventory has little influence on the final steady state, the slightly higher final inventory suggests very small errors or uncertainties in one or more model parameters. The direction of difference suggests there are errors that introduce too much loading and/or production in sediment, or yield too little removal and/or degradation.

However, in contrast to the approximately constant sediment inventory in the base case forecast, the base case methylmercury inventory in the water column decreased about 30%, from an initial water column mass of 0.53 kg, to a final steady-state of only 70% of that (0.37 kg). Although a 30% difference may not be unreasonable for a greatly simplified model of a complex system, the net direction and moderate magnitude of the difference indicates there are factors that might be improved. Some of these factors can be identified through testing the sensitivity of the model to various parameters.

3.3 Sensitivity testing

Given various linkages between water and sediment processes in the model, it is difficult to know a priori how much specific parameter changes will affect overall model response. For example, although increasing loads may help the water inventory match the ambient condition more closely, it might also exacerbate the excess of methylmercury in the steady-state relative to the current ambient condition. Model forecast runs were performed testing various model parameters threefold higher and lower (about an order of magnitude). The methylmercury steady-state inventory of these scenarios relative to the base case steady state were compiled and expressed as the response of the model relative to the base case, compared to the relative difference in the input variable to its base case rate, or magnitude (i.e. a local sensitivity), namely:

Response ratio = (ΔOutput/OutputBASE) / (ΔInput/InputBASE)

where ΔInput/InputBASE is the change in the input parameter divided by the base case input for that parameter, and ΔOutput/OutputBASE is the change in the output (steady state mass of methylmercury), relative to the output in the base case.

There are two components to the output, separately tracking steady state mass in the sediment and in the water column compartments. Although each compartment mass was somewhat linked through modeled exchange and repartitioning among phases, some parameters have a more direct influence on one compartment or the other.

The input parameter factors in Table 4 are listed in order of the magnitude of their effect on the sediment response. A positive ratio (>0%) indicates than an increase in the input parameter yields an increase in the steady-state mass. A negative ratio indicates an inverse relationship. The factors that most influenced the steady-state sediment methylmercury inventory were the sediment methylation and demethylation rates. The final steady-state concentration in sediment was nearly directly related to the changes in these parameters. This result was expected, as these sediment processes far outpaced all others in the base case scenario, and was similar to the findings in a mass budget for Cheasapeake Bay (Mason, Lawson et al. 1999).

The steady state methylmercury mass in the sediment dwarfed the quantity in water, so given the daily uniform mixing and equilibrium partitioning assumptions of the model, not surprisingly those sediment factors also had the greatest influence on water concentrations. Conversely, factors that had moderately large influence on water column inventory (e.g. water demethylation rate, partitioning coefficient, particle settling rate) not surprisingly had only small to nearly no effect on sediment methylmercury inventory.

An increase in the steady-state suspended sediment concentration naturally increased the water column MeHg inventory, as the partitioning coefficient indicates higher concentrations per unit mass in the solid phase compared to the dissolved phase. Increase in external loads, which is modeled as input into the water column, also would be expected to increase primarily the water column concentration; water column processes such as demethylation, volatilization, and tidal flushing/outflow would remove much of the daily load before it could impact the sediment. The residual portion of the load that would settle to the sediment would be only about 20% of the water inventory (given a base case settling velocity of 1 m/day and average water column depth of ~5 m), which at steady-state is much smaller than the sediment inventory and thus has little impact on the latter. Similarly, other model effects on the water column steady-state intuitively make sense. Increases in net outflow and tidal flushing ratio decrease the turnover time of the Bay water volume, exporting a greater proportion of methylmercury in the water column.

Some model responses are counterintuitive, but make sense considered in the larger context of the steady-state assumptions of the model. For example, one would expect that increasing the particle settling rate would tend to pull methylmercury out of the water column and thus decrease the water inventory. However, the steady-state assumption of the model means that the increased particle settling must either be offset by increased resuspension, or increased burial of sediment, in order to maintain a conservation of mass of solids in the mixed surface sediment layer, i.e.

Settling – resuspension = net accretion = burial

This linkage is also seen in the effect of the burial rate; as the burial rate is increased without changing the settling rate, the requirement for steady-state solids mass in the surface sediment layer means that resuspension solids flux must decrease to offset sediment loss via burial (Figure 4). At the extreme, where burial rate exceeds the particle settling rate, the model output enters an imaginary space, where resuspension becomes negative and water solids (and methylmercury) mass could become less than zero.

Model refinement

There are numerous parameters in the model with significant uncertainties due to the limited spatial and temporal extent of the data used in their derivation. Although the model could be tuned to optimize the steady-state output to better represent the current ambient average state (especially in the water column, where the modeled base case assumptions lead to predicted concentrations lower than the average ambient concentrations) these adjustments would generally represent non-unique solutions. For example, given the nearly direct relationship of both methylation and demethylation rate on sediment methylmercury, any adjustment of the sediment methylation rate upward or downward, so long as it were matched by an opposite proportional adjust of the sediment demethylation rate, would result in a steady-state outcome virtually identical to the base case. No attempt was made to identify a set of inputs that would be a “best fit” of the model to existing ambient data, as many combinations of adjustments to other parameters could lead to multiple similar non-unique, and sometimes absurd, solutions.

Given spatial and temporal variability in the Bay, efforts to refine the model would be best spent on improving the spatial and/or temporal specificity the model. Although on a Bay-wide scale external loads have little impact on the sediment methylmercury budget, at smaller spatial scales and on a shorter time scale external loads may have a similar magnitude of impact on sediment concentrations as in-situ production. A more spatially explicit multi-box model of PCB fate in the Bay has already been constructed, resulting in modeled hindcast PCB distributions more in line with regional differences found in ambient monitoring. However, increased model detail places increased data demands, which may be difficult to meet, particularly for biologically mediated processes such as methylation and demethylation.

Stakeholders in the RMP have recently engaged in developing a Mercury Management Strategy, with the objective of identifying whether and how the mercury contamination problem in the Bay might be managed through actions in particularly sensitive components of the food web, localized areas on the Bay margin and /or particular time periods. Through such a strategy, it is hoped that management actions can be applied most effectively.

In the one box model presented here, processes occurring at small spatial and temporal scales that may are relevant to methylmercury fate and biological uptake were simplified to a Bay-wide average basis. The strength of the model as it currently stands was in the building of an inventory of mass loads, and rates and magnitudes of a suite of relevant modeling parameters. If monitoring efforts indicate potential smaller scale problems that could be more easily managed, the increased data collection demands of modeling to understand the ecosystem response at those scales may be warranted, as the ability to tailor management actions to specific problems areas may prove to be most cost effective.

4. Conclusions

The mass budget of methylmercury using a simple one-box model presented here represents a starting point towards a better integrated regional understanding of the sources and fate of methylmercury. The current model was useful as a framework for building an inventory of mass loads and rates for a suite of relevant modeling parameters, for the most part derived from existing local data. The limitations of existing local data also have also been shown though, as there is considerable uncertainty in the derivation of current loads and rates from extrapolating spatially and temporally limited monitoring data. Additionally application of a steady-state one-box model to represent a heterogenous dynamically evolving ecosystem presents major limitations, as methylmercury processes relevant to biological processes at the base of the food web were modeled only on a Bay-wide average basis. Nonetheless, the exercise here represents the current best integration of the state of knowledge for MeHg, an ephemeral pollutant species that is of major concern for SF Bay regional ecosystem managers. Should the need be identified, this initial effort points the way that smaller spatial and temporal scales could be modeled more robustly, provided that appropriate more detailed local information become available. The sensitivity and rapid response of the model to key parameters such as in-situ methylation and demethylation rates suggest that there may be management approaches that could control MeHg in shorter time frames, compared to reductions in total mercury, which may take decades to change even if all external loads are eliminated, given the large inventory already in place in the Bay and slow loss processes.

5. Acknowledgments

The authors would like to acknowledge the Regional Monitoring Program for Water Quality in San Francisco Estuary for financial and technical support; San Francisco Estuary Institute (SFEI) field and data management staff for collection and publication of ambient concentration data; Rob Mason, Jay Davis, xxx, yyy, zzz for the thoughtful review of this manuscript; the Contaminant Fate Workgroup of the RMP for technical guidance and thoughtful review; USGS and DWR for runoff data in the Delta and local watersheds; Bay Area Clean Water Agencies and the San Francisco Bay Regional Water Quality Control Board for information on wastewater discharges; and the numerous researchers in the various studies cited. Without these, this mass budget would not have been possible.

Figures:

Figure 1 – Location Map – circles = sediment stations, diamond = water stations (2002-2006). The shaded area on the inset map represents the Sacramento River and San Joaquin River watersheds, which together comprise approximately 37% of California’s surface area.

Figure 2 – Hindcast model results with zero and 10x the current MeHg inventory as initial state mass, using loading rates at the current best estimate.

Figure 3 – Forecast trajectory of MeHg mass in Bay water and sediment for loading rates base case, 1/3x, and 3x the current best estimate. The dashed lines indicate the ±95% confidence intervals of the mean estimates of current MeHg inventories in Bay water and sediment.

(note to John O the 95% CI is just a raw spatially unweighted one for all the Bay sites 2002-2006- is this close enough to what was in the PBDE paper?  Not that it’s central to any conclusions but nice for all the 1 boxes to be somewhat consistent).

Settling

Reuspension

Burial

Mixed sediment layer

Figure 4. Conceptual Diagram of Mixed Sediment Layer Mass Balance

Table 1 – Summary of estimated daily methylmercury loads (g/day) to San Francisco Bay.

| Loading Pathway | Estimated load |
| --- | --- |
| Delta | 9.8 |
| Local Watersheds | 6.2 |
| Net wetland discharge | 5.3 |
| Wastewater | 0.79 |
| Atmospheric deposition | 0.10 |
| Total | 22.2 |

Table 2. Key Parameters Used in One-Box Model of Methylmercury in San Francisco Bay.

| Parameter | Magnitude | Data Source |
| --- | --- | --- |
| Bay freshwater inflow (m3/s) | 820 | CA Dept. Water Resources |
| Tidal/fresh flow ratio | 3.75 | (Connolly, Ziegler et al. 2005) |
| Degradation rate in water (1/day) | 0.1 | (Yee, Collins et al. 2008) |
| Degradation rate in sediment (1/day) | 0.083 | (Marvin-DiPasquale, Agee et al. 2003) |
| Methylation rate in sediment (ng/g/day) | 0.11 | (Marvin-DiPasquale, Agee et al. 2003) |
| Bay average water MeHg (pg/L) | 95.7 | RMP 2002-2006 |
| Bay average Sediment MeHg (µg/kg) | 0.558 | RMP 2002-2006 |
| Pacific Ocean Water MeHg (pg/L) | 8 | (Choe and Gill 2003) |
| Water column partitioning Kd (l/kg) | 12500 | RMP 2000-2006 |
| Porewater partitioning Kd (l/kg) | 45700 | (Choe, Gill et al. 2004) |
| Sediment burial rate (cm/y) | 0.83 | (Fuller, van Geen et al. 1999) |
| Water-side evaporation coefficient (m/day) | 1.5 | (Hoff, Strachan et al. 1996) |
| Air-side evaporation coefficient (m/day) | 0.26 | (Hoff, Strachan et al. 1996) |
| Water-sed diffusion coefficient (m/day) | 0.001 | (Choe, Gill et al. 2004) |

Table 3. SF Bay Steady-State MeHg Inventories, with Magnitudes of Loading, Exchange, and Loss Processes, Model Base Case

| Model Component | Magnitude |
| --- | --- |
| Mass in water (kg) | 0.38 |
| External load (kg/day) | 0.024 |
| Outflow past Golden Gate | 0.023 |
| Degradation in water | 0.0075 |
| Sediment to water exchange | 0.0064 |
| Biological uptake into fish | .0001 |
| Volatilization | <0.0001 |
| Mass in sediment (kg) | 31.0 |
| Methylation in sediment (kg/day) | 1.82 |
| Degradation in sediment | 1.80 |
| Burial in sediment | 0.0074 |
| Sediment to water exchange | 0.0064 |

Table 4. Model Response to Input Parameters (100% = Direct Response)

| Input Parameter | Sediment | Water |
| --- | --- | --- |
| Sediment methylation rate | 99.3% | 66.0% |
| Sediment demethylation rate | -96.5% | -64.1% |
| Suspended sediment concentration | -0.8% | 49.3% |
| External load | 0.6% | 31.1% |
| Long term net outflow | -0.5% | -23.5% |
| Tidal flushing ratio | -0.4% | -18.6% |
| Water column Kd | 0.4% | -21.8% |
| Particle settling rate | -0.3% | 16.0% |
| Sediment burial rate | -0.2% | -11.0% |
| Water demethylation rate | -0.2% | -9.7% |
| Ocean methylmercury concentration | 0.1% | 3.2% |
| Sediment/water transfer velocity | 0.0% | 0.0% |
| Porewater Kd | 0.0% | 0.0% |
| Water temperature | 0.0% | 0.0% |
| Initial Bay methylmercury concentration | 0.0% | 0.0% |
| Henry's Law constant | 0.0% | 0.0% |
| Air/water mass transfer coefficient | 0.0% | 0.0% |

References