# Proposal to develop a highly-resolved three-dimensional model of South Bay

Investigators: John Oram (SFEI), Ed Gross (Consultant), Mark Stacey (UCB), Rusty Holleman (UCB)

### OVERVIEW

RMP staff are currently drafting a modeling strategy which aims to develop a capacity to predict the effect of different management alternatives on loads from watersheds, the recovery of contaminated areas on the Bay margin, threats from emerging contaminants, and the recovery of the Bay as a whole.  This capacity will be gained through the development of conceptual and numeric models of the physical, chemical, and biological processes governing the fate of water, sediment, and pollutants of concern in San Francisco Bay and its associated watersheds.  RMP modeling will allow managers to predict, prioritize, and optimize the impacts of actions aimed at improving water quality. The overarching goal of the RMP, and the intent of the RMP modeling strategy, is to provide the information needed to support water quality management decisions.

Modeling San Francisco Bay on a flexible grid represents the best potential for addressing the management questions for the Bay and Bay margins. A key advantage of a flexible grid is its ability to increase ‘accuracy’ in dynamically important areas (through grid refinement) and reduce it in other areas. Modeling the Bay with a flexible model would thus implicitly allow for the interaction of fine-scale processes occurring at the Bay Margins with the larger-scale processes of the Bay proper.

There are two basic options for modeling the Bay and its margins on a flexible grid: 1) purchase commercial modeling software, or 2) participate in the development of an open-source community model.  Commercial models are widely used, generally by consulting firms, and are easier to set-up than their open-source counterparts.  The main drawback of commercial models is that their code is not available for testing.  As such, these models are sometimes referred to as black boxes.  The main advantage of open-source models is that they represent the state-of-the-art of environmental modeling.  On the other hand, this state-of-the-art aspect of open-source models can also make model development slow in comparison to commercial software.

Researchers from Stanford and UC Berkeley are currently developing a model of San Francisco Bay with special emphasis on the South Bay Salt Pond restoration efforts. The model (named SUNTANS) is being developed under the open-source software development paradigm. SUNTANS development has been slow and it is uncertain when a public version will be released. Development may be slowed even more due to the California budget crisis. The RMP should consider whether it wants to provide short-term funding of the SUNTANS model. By doing so, the RMP would be able to specify several SUNTANS applications that are directly relevant to the priority management questions outlined in the modeling strategy document.

Here, we propose a project to develop a South Bay SUNTANS grid that includes local tributaries and Bay margins. Depending on the level of funding, model results would be used to drive an offline particle-tracking model to map the zone of influence of local tributaries.

APPLICABLE RMP OBJECTIVES AND MANAGEMENT QUESTIONS

- Are chemical concentrations in the Estuary potentially at levels of concern and are associated impacts likely?

- What are the concentrations and masses of contaminants in the Estuary and its segments?

- What are sources, pathways, loadings, and processes leading to contaminant-related impacts to the Estuary?

- What are the projected concentrations, masses, and associated impacts of contaminants in the Estuary?

Subtask Descriptions

To be developed further in coordination with RMP stakeholders and project investigators

- Develop flexible grid model of South Bay with emphasis on select tributaries (e.g., Guadalupe River) and margins (e.g., San Leandro Bay).

- Perform calibration of hydrodynamic model

- Use hydrodynamic results to drive particle tracking model of select tributaries and/or margins

- Prepare progress report on model development and preliminary results

Budget and timeline

We aim to have this project complete within one year of funding.

| Mark Stacey and Rusty Holleman | UCB |  |
| --- | --- | --- |
| Ed Gross | Consultant |  |
| RMP Staff |  |  |
| Total |  | $100k |